Wireless Display Dongle

Quick Guide

- Connect

  - Please connect to the TV’s HDMI and USB.

TV （BACK）

HDMI

USB

Wecast

USB

MODE

  - TV automatically switches to HDMI mode. If not switching, set the TV switch to HDMI mode.

Mode:  DLNA Mode

Name:  Wecast-123

Status:  Connected to AP

LAN:    192.168.49.1

SSID:    Wecast

WAN:   192.168.1.168

XXX

HDMI

XXX

XXX

- Note：

  - Unplug the power line to save energy for long time if not using.

  - Place the device in a clear environment to ensure the dongle, Android (Apple) device, WIFI hot spot can build high bandwidth, low latency wireless connections.

  - If using WIFI hotspot as bridge to share your screen to WIFI DISPLAY dongle, please ensure the hotspot has a strong performance to provide high quality wireless connection to handheld devices and dongle；

  - The latest version will automatically update, just keep the device connected to the Internet.

- Quick guide for iOS

  - Short press the MODE key and switch to DLNA mode.

  - Open WIFI on your iOS device and connect to WIFI DISPLAY dongle ( Name: Wecast-xxxxxx, Password:  12345678 ) or your WIFI hotspot ( Please refer to 4.4, 4.5 and set the the WIFI DISPLAY dongle connect to WIFI hotspot first ).

iOS6

Airplay

iPhone

Wecast-123√

Mirroring

Wi-Fi Setup

XXXX

Wecast-123

XXX

XXXXX

iOS7

AirDrop

AirPlay

  - Open Airplay Mirroring switch on your iOS device and choose WIFI DISPLAY dongle, TV will share your screen immediately. Open Airplay Mirroring switch and choose iOS device the screen share will stop.

  - When the first use of this dongle please set it connected to WIFI hotspot.

    - Follow 4.2 and connected your iOS device to WIFI DISPLAY dongle ( Name: Wecast-xxxxxx, password: 12345678).

    - Open web browser on your iOS device and visit 192.168.49.1, choose the WIfi setup, connect to your WIFI hotspot.

    - After connected successfully, TV automatically shows SSID and IP address of the WIFI hotspot.

Mode:  DLNA Mode

Name:  Wecast -123

Status:  Disconnected from AP

LAN:   192.168.49.1

SSID:

WAN:

Mode:  DLNA Mode

Name:  Wecast -123

Status:  Connected to AP

LAN:   192.168.49.1

SSID:   Wecast

WAN:  192.168.1.168

  - You can use screen share in two ways. Please make sure the WIFI hotspot can provide high quality wireless connection to handheld devices and dongle if choosing way 2.

WIFI hotspot

WIFI hotspot

Wecast

Wecast

Way 2

Way 1

- Quick guide for Android

  - Short press the MODE key and switch to Miracast mode.

  - Open Miracast switch on your Android device and choose Wecast -XXXXX, the device will automatically connect.

Available device

Wecast-123

Press to connect  XXXXXXXX

Setup

WLAN

Display

Screen share

  - After the connection is successful, TV will automatically share your screen.

Mode:  Miracast Mode

Name:  Wecast -123

Status:  Connection in progress…

Device:

MAC:

PORT:

Mode:  Miracast Mode

Name: Wecast -123

Status:  Waiting for connection…

Device:

MAC:

PORT:

  - Miracast only provide screen share between your Android device and Wecast dongle. Please connect your Android device to WIFI hotspot if need Internet service.

Wecast

WIFI hotspot

Internet service

Wireless screen share

  - There are different names of Miracast on different Android devices, such as WIFI Display、WLAN Display、Wireless Display、Allshare Display、Allshare Cast etc.

  - The way to open Miracast switch on different Android devices is different, you can find it on WLAN、WIFI Setup、Display and so on in the setting menu of your device.

Notice : We will update our product without any notice.